Ultrasonic Distance Meter uses a Ultrasonic sensor, a buzzer and a Arduino board to measure distance of a obstacle/object from the sensor.

When too near a buzzer will sound. Set Serial Monitor to 9600 Baud

const int trigPin = 8;

const int echoPin = 9;

const int buzzerPin = 7;

void setup() {

Serial.begin(9600);

pinMode(trigPin, OUTPUT);

pinMode(echoPin, INPUT);

pinMode(buzzerPin, OUTPUT);

}

void loop() {

// Send pulse

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

// Read echo

long duration = pulseIn(echoPin, HIGH);

float distance = duration * 0.034 / 2;

Serial.print("Distance: ");

Serial.print(distance);

Serial.println(" cm");

// Trigger buzzer if distance < 30 cm

if (distance > 0 && distance < 10) {

digitalWrite(buzzerPin, HIGH); // Beep!

} else {

digitalWrite(buzzerPin, LOW);  // Silent

}

delay(500);

}

Connections: 
Trig Pin: D8
Echo Pin: D9
Buzzer: D7