Using One IR sensor
// IR sensor and motor/buzzer pins connections

 IR1  → Arduino D2

IN1 → Arduino D4

  IN2 → Arduino D5

  IN3 → Arduino D6

  IN4 → Arduino D7

  ENA (Enable A) → Either 5V (always on) or PWM pin if you want speed control (usually D9, D10).

  ENB (Enable B) → Same as above. //

const int irSensor = 2;

const int leftMotorForward = 4;

const int leftMotorBackward = 5;

const int rightMotorForward = 6;

const int rightMotorBackward = 7;

const int buzzerPin = 8;

void setup() 
{

pinMode(irSensor, INPUT);

pinMode(leftMotorForward, OUTPUT);

pinMode(leftMotorBackward, OUTPUT);

pinMode(rightMotorForward, OUTPUT);

pinMode(rightMotorBackward, OUTPUT);

pinMode(buzzerPin, OUTPUT);

}

void loop() 
{

int obstacle = digitalRead(irSensor); // LOW = obstacle

if (obstacle == HIGH) {

// No obstacle – move forward

forward();

digitalWrite(buzzerPin, LOW);

} else {

// Obstacle detected – turn right & sound buzzer

turnRight();

digitalWrite(buzzerPin, HIGH);

delay(500); // turn time

}

delay(50);

}

// === Motor control functions ===

void forward() 
{

digitalWrite(leftMotorForward, HIGH);

digitalWrite(leftMotorBackward, LOW);

digitalWrite(rightMotorForward, HIGH);

digitalWrite(rightMotorBackward, LOW);

}

void turnRight() 
{

digitalWrite(leftMotorForward, HIGH);

digitalWrite(leftMotorBackward, LOW);

digitalWrite(rightMotorForward, LOW);

digitalWrite(rightMotorBackward, HIGH);

}

void stopMotors() {

digitalWrite(leftMotorForward, LOW);

digitalWrite(leftMotorBackward, LOW);

digitalWrite(rightMotorForward, LOW);

digitalWrite(rightMotorBackward, LOW);

}

Using 2 IR Sensors

// Pin assignments

 IR1  → Arduino D2

 IR2  → Arduino D3

IN1 → Arduino D4

  IN2 → Arduino D5

  IN3 → Arduino D6

  IN4 → Arduino D7

  ENA (Enable A) → Either 5V (always on) or PWM pin if you want speed control (usually D9, D10).

  ENB (Enable B) → Same as above. //

const int leftSensor = 2;

const int rightSensor = 3;

const int leftMotorForward = 4;

const int leftMotorBackward = 5;

const int rightMotorForward = 6;

const int rightMotorBackward = 7;

const int buzzerPin = 8;

void setup()             {

pinMode(leftSensor, INPUT);

pinMode(rightSensor, INPUT);

pinMode(leftMotorForward, OUTPUT);

pinMode(leftMotorBackward, OUTPUT);

pinMode(rightMotorForward, OUTPUT);

pinMode(rightMotorBackward, OUTPUT);

pinMode(buzzerPin, OUTPUT);

}

// Reads sensor 5 times with tiny delays and returns stable result

int stableReadIR(int pin) {

int count = 0;

for (int i = 0; i < 5; i++) {

if (digitalRead(pin) == LOW) count++;

delay(2);  // debounce delay

}

return (count >= 3) ? LOW : HIGH;

}

void loop() {

int leftObstacle = stableReadIR(leftSensor);   // LOW = Obstacle

int rightObstacle = stableReadIR(rightSensor); // LOW = Obstacle

if (leftObstacle == HIGH && rightObstacle == HIGH) {

// No obstacles - move forward

forward();

digitalWrite(buzzerPin, LOW);

}

else {

// Obstacle detected

digitalWrite(buzzerPin, HIGH);

if (leftObstacle == LOW && rightObstacle == HIGH) {

turnRight();

}

else if (leftObstacle == HIGH && rightObstacle == LOW) {

turnLeft();

}

else {

stopMotors();

}

}

delay(50); // brief delay for stability

}

// === Motor control functions ===

void forward() {

digitalWrite(leftMotorForward, HIGH);

digitalWrite(leftMotorBackward, LOW);

digitalWrite(rightMotorForward, HIGH);

digitalWrite(rightMotorBackward, LOW);

}

void turnLeft() {

digitalWrite(leftMotorForward, LOW);

digitalWrite(leftMotorBackward, HIGH);

digitalWrite(rightMotorForward, HIGH);

digitalWrite(rightMotorBackward, LOW);

}

void turnRight() {

digitalWrite(leftMotorForward, HIGH);

digitalWrite(leftMotorBackward, LOW);

digitalWrite(rightMotorForward, LOW);

digitalWrite(rightMotorBackward, HIGH);

}

void stopMotors() {

digitalWrite(leftMotorForward, LOW);

digitalWrite(leftMotorBackward, LOW);

digitalWrite(rightMotorForward, LOW);

digitalWrite(rightMotorBackward, LOW);

}